| AdiNexus IoT Gateway V2.0 LITE • User Manual For End-Customer Use — Zambia Deployment | Model: ADN-GW-V2L Rev: 1.0 2025 |
| --- | --- |

This document provides installation, connection, and operational guidance for the AdiNexus IoT Gateway V2.0 Lite. Read all instructions before powering on the device. Keep this manual in a safe place for future reference.

1.  Package Contents

| 1 × | AdiNexus IoT Gateway V2.0 Lite unit |
| --- | --- |
| 1 × | Power adapter (12 V DC, 2 A) |
| 1 × | 4G / LTE antenna (SMA connector) |
| 1 × | Wi-Fi / BLE antenna (SMA connector) |
| 1 × | Ethernet patch cable (1 m) |
| 1 × | Quick-start card |

2.  Product Overview

The AdiNexus IoT Gateway V2.0 Lite is an industrial-grade edge computing gateway designed for smart-city, infrastructure monitoring, and remote IoT deployments. It bridges field sensors and cloud platforms using MQTT over 4G/LTE, Wi-Fi, or Ethernet, with built-in OTA firmware update support and VPN capability.

3.  Technical Specifications

| Processor & Memory |  |
| --- | --- |
| Processor | Espressif ESP32-WROVER-IE (Xtensa dual-core LX6, up to 240 MHz) |
| Flash Memory | 16 MB (SPI Flash) |
| RAM | 8 MB PSRAM + 520 KB SRAM |
| Operating System | ESP-IDF (FreeRTOS-based) |
| Connectivity |  |
| Cellular | 4G LTE / 3G / 2G (CAT-1), SIM7600 module, Nano SIM |
| Wi-Fi | IEEE 802.11 b/g/n (2.4 GHz), up to 150 Mbps |
| Bluetooth | Bluetooth 4.2 BR/EDR + BLE 5.0 |
| Ethernet | 10/100 Mbps, RJ-45 connector |
| MQTT Broker | Supports HiveMQ, Mosquitto, AWS IoT Core, Azure IoT Hub |
| VPN | WireGuard VPN client (built-in) |
| I/O Interfaces |  |
| Serial (UART) | 2 × RS-232 / RS-485 (configurable) |
| GPIO | 8 × digital I/O (3.3 V logic) |
| Analog Input | 2 × ADC (12-bit, 0–3.3 V) |
| USB | 1 × Micro-USB (firmware flash / serial debug) |
| SIM Slot | 1 × Nano SIM (push-push) |
| Power |  |
| Input Voltage | 9–24 V DC (recommended 12 V DC) |
| Power Connector | 2-pin terminal block / DC barrel jack |
| Consumption | Typical 1.2 W idle; peak 4.5 W (cellular TX) |
| Reverse Polarity | Protected |
| Physical |  |
| Dimensions | 110 × 75 × 28 mm (L × W × H) |
| Weight | ~185 g (without antennas) |
| Enclosure | ABS plastic, DIN-rail mountable |
| Operating Temp | −10 °C to +60 °C |
| Storage Temp | −20 °C to +70 °C |
| Humidity | 10–90 % RH (non-condensing) |
| Protection | IP30 |
| Software & Cloud |  |
| Config Method | Cloud-based (MQTT), local binary, BLE provisioning |
| OTA Updates | Over-the-Air firmware update via MQTT |
| Protocols | MQTT, HTTP/HTTPS, Modbus RTU/TCP, UART, BLE GATT |
| Data Security | TLS 1.2 / 1.3, token-based auth, encrypted config |

4.  LED Status Indicators

The gateway has three LED indicators on the front panel. Use the table below to identify the current operating state without requiring logs or a computer connection.

| P1 — WiFi LED (Green/Amber) | P2 — Red LED (Status / Error) | P3 — Blue LED (Connectivity) |
| --- | --- | --- |

| Device State | P1 WiFi LED | P2 Red LED | P3 Blue LED | Visual Description |
| --- | --- | --- | --- | --- |
| Boot / Restart | — | Steady ON | Steady ON | Both solid (Red + Blue) |
| Init Failed | — | Blink 3s | OFF | Red slow blink only |
| Config from Cloud | — | Steady ON | Blink 2s | Red solid + Blue blink |
| Config from Binary | — | Blink 2s | Steady ON | Red blink + Blue solid |
| GET_SETTINGS cmd | — | Blink 2s | Steady ON | Red blink + Blue solid |
| OTA in Progress | — | Blink 2s | OFF | Red blink only |
| MQTT Connected | — | OFF | Steady ON | Blue solid only |
| MQTT Disconnected | — | Steady ON | OFF | Red solid only |
| VPN Mode | — | Alt. 1s ON/OFF | Alt. 1s OFF/ON | Red→Blue→Red (2s cycle) |
| BLE Mode | — | Blink 3s | Blink 3s | Both blink in sync |
| WiFi Connected | Steady ON | — | — | WiFi LED solid |
| WiFi Disconnected | Blink 2s | — | — | WiFi LED blink |

ℹ LED Tip Normal operation: P3 Blue LED solid (MQTT Connected). If only P2 Red is solid, the device has lost its MQTT/cloud connection — check SIM card, antenna, and network. If P2 blinks slowly (3 s) and P3 is OFF, initialisation has failed — power-cycle the device.

5.  Quick Setup Guide

Step 1 — Insert SIM Card

Power off the unit. Open the SIM slot cover on the side panel. Insert a Nano SIM (4G/LTE capable) with the gold contacts facing down. Close the cover securely.

Step 2 — Attach Antennas

Screw the 4G antenna onto the LTE port and the Wi-Fi/BLE antenna onto the WLAN port. Finger-tight only — do not over-torque.

Step 3 — Connect Power

Connect the supplied 12 V DC adapter to the power input. Both P2 (Red) and P3 (Blue) LEDs will glow solid during boot. Wait up to 60 seconds for the device to initialise and connect to the cloud.

Step 4 — Verify Cloud Connection

Once connected, P3 Blue LED will remain solid and P2 Red will turn OFF. The device is now publishing data to the MQTT broker. Log in to your AdiNexus cloud dashboard to confirm device registration.

6.  Safety & Regulatory Notes

⚠ Important Safety Information Do not expose the device to rain, moisture, or direct sunlight. Ensure adequate ventilation — do not block the enclosure vents. Use only the supplied power adapter. Using an incorrect adapter may damage the unit and void the warranty. In case of unusual smell, smoke, or heat, disconnect power immediately and contact your distributor. This device contains a lithium backup battery — do not disassemble or incinerate.

7.  Support & Contact

| Website | https://adinexus.com |
| --- | --- |
| Email Support | support@adinexus.com |
| Distributor | Please contact your local AdiNexus-authorised distributor in Zambia |
| Warranty | 12 months from date of purchase (hardware defects) |
| Document Rev | v1.0 ¦ 2025 |

© 2025 AdiNexus Technologies. All rights reserved. Specifications subject to change without notice.